G-7000 MODEL

POLYURETHANE INJECTED

GARAGE DOOR

GALVANIZED STEEL

26 GAUGE, door thickness 35 mm (1 3/8")

GARAGA INC.								SECTION: 083613

FILE: ___________________						Page: _______________

OVERHEAD RETRACTABLE DOORS

WITH METAL PANELS ON HINGES

PART 1: OVERVIEW

1.1	RELATED WORK

The author shall give all the necessary references.

1.1.1	Electrical connections, water tightness of openings, and so on.  Remember: all electrical connections and installations must be done by a qualified electrician as recommended by the manufacturer of the electric garage door operators.  (Section 16)

1.1.2	Preparation of the opening of the garage door (Section 5).

1.1.3	These construction specifications and shop drawings are applicable to Garaga Inc products only.

1.2	SHOP DRAWINGS

1.2.1	Submit the shop drawings as per the instructions outlined in Section 01300.

1.2.2	The shop drawings shall include all the following information: the type of materials, the type of opening mechanism, the required tolerances, the electrical connections, the structural fastenings executed by the general contractor, and the suitable matching with neighboring materials.

1.3	MAINTENANCE RECORDS

1.3.1	Provide the necessary instructions to ensure proper operation and maintenance of all the hardware components for the doors as well as the electric garage door operators, and include these instructions with the manual on use and maintenance described in Section 01300.

1.4	QUALIFICATIONS

1.4.1	The manufacturer of materials shall be registered in accordance with ISO-9001 (2000 version) standards.  The installation of this section shall be fabricated by a manufacturer with a minimum of 7 years experience in the actual production of the specified products.

1.4.2	The installation shall be executed by a company approved by the garage door manufacturer as an installer, using skilled installers experienced on this work.

PART 2: GARAGE DOORS

2.1	CALCULATION CRITERIA

2.1.1	The doors and the hardware system shall be designed to meet or exceed the industry standard for windload (Door & Access Systems Manufacturer Association).

(Note: for doors wider than 5.5 m (18’), or high wind situations, consult our engineering service).

2.1.2	The doors shall have a thermal resistance factor of RSI 2.1 (k = 0.476 W/m2K)

2.1.3	The doors, the tracks, and the springs shall be designed to withstand at least ________operation cycles per year and _________total cycles over their useful life.

Note: the number of spring cycles can vary from 10 000 to 100 000 cycles. However, you must realize that you cannot get the high cycles for every size of door. If you want the best operating system, with the highest cycles possible, please specify “springs shall be designed for the maximum operation cycles”.

2.2	MATERIALS

2.2.1	Enameled steel (26 gauge on both sides)

26 gauge DQSK galvanized steel sheet in accordance with A.S.T.M. A653-97 & A.S.T.M. 653M-97, has G.60 coating, in manufacturers standard colour.  The galvanized steel sheet has a zinc coating with a minimum of 180 g/m2 (0.04 lb/in2).  The polyester paint finish, 2 layers, conforms to standard A.S.T.M. A653-97 & A.S.T.M. A653M-97 and has a thickness of 1.0 mils.  The surface of the steel sheet is woodgrain with decorative horizontal grooves.

2.2.2	Insulation

High-pressure, CFC-Free, polyurethane foam has been injected between the walls of each panel.  Its density is 40.4 Kg/m3 (2.5 lb/ft3) with a thermal resistance factor of RSI 1.6 per 25 mm (1”) of thickness.  The total insulation factor is R-12, RSI 2.1 (k = 0.476 W/m2K) (A.S.T.M. standard C-518-91). This insulation is in accordance with standards ONGC 51-GP-21M and 51.26-M86.

2.2.3	Reinforcements

Hinge fasteners, panel reinforcement fastener for mechanical lifting device and handles: steel reinforcement 14 gauge.

2.2.4	Panel ends

A block of dry pine (grade 4) is inserted at both ends of each insulated garage door section for the fastening of the lateral hinges.

2.2.5	Regular windows (if applies)

Clear double thermo pane windows shall have a total thickness of 22.2 mm (7/8”).  The 3 mm (1/8") panes are sealed in aluminum extrusions.  The windows are inserted in an expanded PVC frame and factory installed by the manufacturer.

2.3	DOORS

2.3.1	The garage doors shall be of the model G-7000 as made by Garaga Inc.  The panels are shaped with 26 gauge steel and electronically injected with high pressure polyurethane for a total minimum thickness of 35 mm (1 3/8”).

2.3.2	The doors shall have the following sizes and features:

DOORS	            SIZES		  # OF SECTIONS	                  # OF PANES

LOCATION	         (W. by H.)		     WITH PANES		    PER SECTION

__________	___________	________________		_________________

__________	___________	________________		_________________

__________	___________	________________		_________________

2.4	WEATHER TIGHTNESS

2.4.1	Provide and install continuous weatherstripping at the bottom of the lower panel.  The weatherstripping shall be made of a U-shape black PVC extrusion as well as a semi-circular E.P.D.M. rubber tubing.

2.4.2	Inset weatherstripping of flexible and rigid PVC shall be found at the intersection of each panel.  This type of weatherstripping will ensure an efficient thermal barrier as well as double weather tightness in accordance with the following standards: when submitted to a pressure of 0.075 kPa, which is equivalent to winds of 40 km/h, the air infiltration rating as measured using standard A.S.T.M. E-283 shall be of 0.033 liter/second per meter of joint between the door sections.

2.4.3	Provide and install, on the exterior side of the door jambs and lintel, weatherstripping made up of an aluminum extrusion as well as a double-edged strip of arctic vinyl.  This weatherstripping shall be adjustable and come with rigid PVC screw cover.

2.5	OPTIONS (to choose from)

2.5.1	End caps that are made in 16 gauge galvanized steel shall be installed at the end of each section where the hinges are to be fastened.

2.5.2	Exhaust ports of 100 mm (4”) in diameter are factory installed on each door in order to place flexible hoses for evacuating exhaust gases.

PART 3: HARDWARE WITH TORSION SPRINGS

3.1	PRODUCTS

3.1.1	The tracks

The tracks are made of 12 gauge (2.6 mm (0.1”)) galvanized steel of 76 mm (3”).  The horizontal track is reinforced with a steel angle of 50 x 50 mm (2” x 2”)

3.1.2	Hardware

The hinges are made of 13 gauge galvanized steel.  Industrial rollers of 75 mm (3”) with 10 ball- bearings are used.

3.1.3	Struts for large doors (if applies)

Doors measuring in excess of 3759 mm (12'-4") in width will come with 22 gauge galvanized steel horizontal struts, and 13 gauge double hinges at each end.

3.1.4	Type of movement

The movement of the hardware will allow for the most space possible available underneath the door when it is in the open position.

DOORS	    	      TYPE OF MOVEMENT	 	  AVAILABLE SPACE	                LOCATION	       					 	   (FLOOR/CEILING)

____________		     _____________________	           ______________________

____________		     _____________________	           ______________________

____________		     _____________________	           ______________________

3.1.5	Torsion-type springs

The torsion spring lifting system will consist of all the parts and accessories needed for its installation.  All doors weighting more than 454 kg (1000 lb), hardware parts included, must be approved by a professional in installation in regards to the choice of pieces of hardware (drums, galvanized cables, springs, anchor plate, solid shaft of 25 mm (1”)).

3.2	OPTIONS (to choose from)

3.2.1	Track guards

The vertical tracks are protected by track guards (in an L shape 1524 mm x 6.4 mm (5’ x 0.3”) or in a Z shape 1524 mm x 5.0 mm (5’ x 0.2”)) in order to avoid accidental breaking.  Often, track guards are painted with a light colour after the installation.

3.2.2	Chain hoist

Doors of over 3 m (10’) high with manual operation will come equipped with a chain hoist mounted on the wall.

Note: If electric operator, consult specs in the next section.

3.2.3	Pusher springs (to choose from)

For doors exceeding 15 m2 (161 ft2), the standard movement hardware will come with pusher springs at the end of the horizontal tracks in order to prevent the cables from falling off the drums.

3.2.4	"C" bumper springs

For dock doors (ex: 2438 mm x 2438 mm (8’ x 8’)), the vertical lift movement hardware will come with “C” bumper springs at the end of the vertical tracks in order to prevent the cables from falling off the drums.

CHOOSE:	- Trolley-type operators for commercial use

- Jackshaft-type operators

PART 4:	TROLLEY-TYPE OPERATORS FOR COMMERCIAL USE

4.1	PRODUCTS

4.1.1	These are trolley-type electric operators that come equipped with a quick-release device which instantly disconnects the door from the operator to enable manual operation in the event of a power failure.  The system consists of a carriage that slides between dual galvanized steel angle tracks.

Note: that 100 mm (4”) of free space is required between the highest point of the door travel and the ceiling.

4.1.2	The electric motors, control mechanisms, relays, and electrical devices of the operator shall be approved according to CSA and UL standards.

4.1.3	The electrical power supply is of ____________ volts, _______ phase(s) and 60 Hz.

DOORS		         MODEL 		HORSEPOWER	 * TYPE OF

LOCATION								 MOVEMENT

______________	________________	  ___________		____________

______________	________________	  ___________		____________

______________	________________	  ___________		____________

* Standard lift only

4.1.4	The operators come equipped with an instantly reversible motor, which has built-in thermal protection mechanism. The electrical control circuit shall be of 24 volts.

4.1.5	The door shall travel at approximately 200 to 280 mm/sec (7.9 to 11 in/sec).

4.1.6	The control panel for the door operator shall be of the push-button type for Up/Down/Stop control and surface mounted on the inside wall.

4.1.7	A safety mechanism with photocells shall be added to each operator in order to stop and reverse the movement of the door in the event an object cuts off the light beam.

AND / OR

An electro-pneumatic safety device with an instant reverse feature will be added to each operator.  It consists of a rubber hose concealed within the weatherstripping located at the bottom of the door, combined with the electric switches.  This device makes the door stop and go back up as soon as it comes into contact with an object.

4.2	OPTIONS (to choose from)

4.2.1	The door operators identified by the numbers _____ come with a remote control system.

4.2.2	A control station with a key switch for Up/Down/Stop shall be surface mounted on the outside of the building.

4.2.3	All side latches will be equipped with an electrical interlock switch to prevent use of the electric operator when the door is locked.

PART 4:	JACKSHAFT-TYPE OPERATORS

4.1	PRODUCTS

4.1.1	The electric operators shall be of the Jackshaft type and shall come equipped with a built-in chain hoist assembly and with a quick release mechanism to allow for manual operation of the door in the event of a power failure.

4.1.2	The electric motors, control mechanisms, relays, and electrical devices of the operator shall be approved according to CSA and UL standards.

4.1.3	The electrical power supply is of ____________ volts,  _______ phase(s) and 60 Hz.

DOORS		         MODEL 		HORSEPOWER	 * TYPE OF

LOCATION								 MOVEMENT

______________	________________	  ___________		____________

______________	________________	  ___________		____________

______________	________________	  ___________		____________

4.1.4	The operators shall feature a positive lock mechanical brake, an instantly reversible motor, and a thermal protection device against power surges. The electric control circuit is of 24 volts.

4.1.5	The door shall travel at a speed of 200 to 230 mm/sec (7.9 to 11 in/sec).

4.1.6	The control panel for the door operator shall be of the push-button type for Up/Down/Stop control and surface mounted on the inside wall.

4.1.7	A safety mechanism with photocells shall be added to each operator in order to stop and reverse the movement of the door in the event an object cuts off the light beam.

AND / OR

An electro-pneumatic safety device with an instant reverse feature will be added to each operator.  It consists of a rubber hose concealed within the weatherstripping located at the bottom of the door, combined with the electric switches.  This device makes the door stop and go back up as soon as it comes into contact with an object.  It also comes with a 1.5 second delay on reverse feature which helps prevent too much bouncing on the lifting cables when the door is going back up.

4.2	OPTIONS (to choose from)

4.2.1	The door operators identified by the numbers _____ come with a remote control system.

4.2.2	A control station with a key switch for Up/Down/Stop shall be surface mounted on the outside of the building.

4.2.3	All side latches will be equipped with an electrical interlock switch to prevent use of the electric operator when the door is locked.

PART 5: INSTALLATION

5.1	Before starting, make sure that the frames and the fixtures prepared by the general contractor are square.

5.2	Install the doors and the related hardware.

5.3	Apply some touch-up paint to areas where the finish might have been damaged during the mounting.

5.4	Install the electric motors, control devices, push-button control stations, relays, and other electrical equipment needed for operating the door.

5.5	All electrical connections must be done by a certified electrician.

5.6	Adjust all movable parts in such a way as to get good weather tightness against all conditions.

5.7	Adjust the weatherstripping with the exterior jambs in such a way as to get good weather tightness against all conditions.

5.8	Make sure all of the mechanisms that have been installed work properly.

5.9	Clean doors as recommended by the manufacturer, and get rid of all leftover materials and debris found near the openings and the hardware.